Conduct the following experiments on an ARM CORTEX M3 evaluation board using evaluation version of Embedded 'C' & Keil Uvision-4 tool/compiler.

1. Blink an LED with software delay

#include <lpc17xx.h>

void delay_ms(unsigned int ms)

{

unsigned int i,j;

for(i=0;i<ms;i++)

for(j=0;j<20000;j++);

}

/* start the main program */

int main(void)

{

SystemInit();                    /*Clock and PLL configuration*/

LPC_PINCON->PINSEL4 = 0xffffffff;  /*Configure the PORT2 Pins as GPIO;*/

LPC_GPIO2->FIODIR = 0xffffffff;  /*Configure the PORT2 pins as OUTPUT;*/

while(1)

{

LPC_GPIO2->FIOSET = 0xffffffff;     /* Make all the Port pins as high */

delay_ms(10);

LPC_GPIO2->FIOCLR = 0xffffffff;     /* Make all the Port pins as low */

delay_ms(10);

}

}

Output:

In Peripherals  GPIO pins  check pin 2 to see the pins.

2. Blink an LED with delay generated using the SysTick timer.

#include <LPC17xx.h>

/* Systick Register address, refer datasheet for more info */

#define STCTRL      (*( ( volatile unsigned long *) 0xE000E010 ))

#define STRELOAD    (*( ( volatile unsigned long *) 0xE000E014 ))

#define STCURR      (*( ( volatile unsigned long *) 0xE000E018 ))

/*******STCTRL bits*******/

#define SBIT_ENABLE     0

#define SBIT_TICKINT    1

#define SBIT_CLKSOURCE  2     /* 100000000Mhz * 1ms = 1000000 - 1 */

#define RELOAD_VALUE  99999999

#define LED   2 //P2_2

int main (void)

{

SystemInit();

STRELOAD = RELOAD_VALUE;    // Reload value for 100ms tick

/* Enable the Systick, Systick Interrup and select CPU Clock Source */

STCTRL = (1<<SBIT_ENABLE) | (1<<SBIT_TICKINT) | (1<<SBIT_CLKSOURCE);

LPC_GPIO2->FIODIR = (1<<LED);            /* Configure the Led Pin as Output */

while(1)

{

//do nothing

}

}

void SysTick_Handler(void)

{

LPC_GPIO2->FIOPIN ^= (1<<LED); /* Toggle the LED1 (P2_0) */

}

Output:

3. System clock real time alteration using the PLL modules.

#include <LPC17xx.h>

#define CCLKCFG (*(volatile unsigned long *)(0x400FC104))

#define PLL0CON (*(volatile unsigned long *)(0x400FC080))

#define PLL0FEED (*(volatile unsigned long *)(0x400FC08C))

#define PLL0STAT (*(volatile unsigned long *)(0x400FC088))

#define PLL0CFG (*(volatile unsigned long *)(0x400FC084))

// function prototypes

void delay(void);

int main() {

LPC_GPIO2->FIODIR |= 0x0000007C;

//CCLKCFG=0x000000EE; // divider divides by this number plus 1

// Set PLL0 multiplier

PLL0CFG = 0x0015013A; //arbitrary multiply value, divide value left at 1

PLL0FEED = 0x000000AA; // Feed the PLL

PLL0FEED = 0x00000055;

// Turn on PLL0

PLL0CON |= 1<<0;

PLL0FEED = 0x000000AA; // Feed the PLL

PLL0FEED = 0x00000055;

// Wait for main PLL (PLL0) to come up

while ((PLL0STAT & (1<<24)) == 0x00);

// Wait for PLOCK0 to become 1

while ((PLL0STAT & (1<<26)) == 0x00);

// Connect to the PLL0

PLL0CON |= 1<<1;

PLL0FEED = 0x000000AA; // Feed the PLL

PLL0FEED = 0x00000055;

while ((PLL0STAT & (1<<25)) == 0x00); //Wait for PLL0 to connect

while(1) {

LPC_GPIO2->FIOPIN ^= (0x0000007C);

delay();

}

}

void delay(void)

{          //delay function.

int j;                     //loop variable j

for (j=0;j<50000;j++)

{

j++;

j--;                  //waste time

}

}

Output:

In options panel go to debug panel, select option ‘use JLINK’ and change text to DARMP1.sll and on right side TARMP1.sll

Go to peripherals  System clock  PLL 0

When PLL is On , the SYSCLK and PLLCLK are different and PLLCLK>SYSCLK.

PLL Off

#include <LPC17xx.h>

#define CCLKCFG (*(volatile unsigned char *)(0x400FC104))

#define PLL0CON (*(volatile unsigned char *)(0x400FC080))

#define PLL0FEED (*(volatile unsigned char *)(0x400FC08C))

#define PLL0STAT (*(volatile unsigned char *)(0x400FC088))

// function prototypes

void delay(void);

int main() {

LPC_GPIO2->FIODIR |= 0x0000007C;

CCLKCFG = 0x000000FF;

// Disconnect PLL0

PLL0CON &= !(1<<1); // Clears bit 1 of PLL0CON, the Connect bit

PLL0FEED = 0xAA; // Feed the PLL. Enables action of above line

PLL0FEED = 0x55; //

// Wait for PLL0 to disconnect. Wait for bit 25 to become 0.

while ((PLL0STAT & (1<<25)) != 0x00);//Bit 25 shows connection status

// Turn off PLL0; on completion, PLL0 is bypassed.

PLL0CON &= !(1<<0); //Bit 0 of PLL0CON disables PLL

PLL0FEED = 0xAA; // Feed the PLL. Enables action of above line

PLL0FEED = 0x55;

// Wait for PLL0 to shut down

while ((PLL0STAT & (1<<24)) != 0x00);//Bit 24 shows enable status

/****Insert Optional Extra Code Here****

to change PLL0 settings or clock source.

**OR** just continue with PLL0 disabled and bypassed*/

//blink at the new clock frequency

while(1) {

LPC_GPIO2->FIOPIN ^= (0x0000007C);

delay();

}

}

void delay(void)

{          //delay function.

int j;                     //loop variable j

for (j=0;j<5000000;j++)

{

j++;

j--;                  //waste time

}

}

Output:

In options panel go to debug panel, select option ‘use JLINK’ and change text to DARMP1.sll and on right side TARMP1.sll

Go to peripherals  System clock  PLL 0

When PLL is Off , the SYSCLK and PLLCLK are same and PLLCLK=SYSCLK.

4. Using the Internal PWM module of ARM controller generate PWM and vary its duty cycle.

#include <lpc17xx.h>

void delay_ms(unsigned int ms)

{

unsigned int i,j;

for(i=0;i<ms;i++)

for(j=0;j<50000;j++);

}

#define SBIT_CNTEN     0

#define SBIT_PWMEN     2

#define SBIT_PWMMR0R   1

#define SBIT_LEN0      0

#define SBIT_LEN1      1

#define SBIT_LEN2      2

#define SBIT_LEN3      3

#define SBIT_LEN4      4

#define SBIT_PWMENA1   9

#define SBIT_PWMENA2   10

#define SBIT_PWMENA3   11

#define SBIT_PWMENA4   12

#define PWM_1          0 //P2_0 (0-1 Bits of PINSEL4)

#define PWM_2          2 //P2_1 (2-3 Bits of PINSEL4)

#define PWM_3          4 //P2_2 (4-5 Bits of PINSEL4)

#define PWM_4          6 //P2_3 (6-7 Bits of PINSEL4)

int main(void)

{

int dutyCycle;

SystemInit();

/* Cofigure pins(P2_0 - P2_3) for PWM mode. */

LPC_PINCON->PINSEL4 = (1<<PWM_1) | (1<<PWM_2) | (1<<PWM_3) | (1<<PWM_4);

/* Enable Counters,PWM module */

LPC_PWM1->TCR = (1<<SBIT_CNTEN) | (1<<SBIT_PWMEN);

LPC_PWM1->PR  =  0x0;               /* No Prescalar */

LPC_PWM1->MCR = (1<<SBIT_PWMMR0R);  /*Reset on PWMMR0, reset TC if it matches MR0 */

LPC_PWM1->MR0 = 100;                /* set PWM cycle(Ton+Toff)=100) */

LPC_PWM1->MR1 = 50;                 /* Set 50% Duty Cycle for all four channels */

LPC_PWM1->MR2 = 50;

LPC_PWM1->MR3 = 50;

LPC_PWM1->MR4 = 50;

/* Trigger the latch Enable Bits to load the new Match Values */

LPC_PWM1->LER = (1<<SBIT_LEN0) | (1<<SBIT_LEN1) | (1<<SBIT_LEN2) | (1<<SBIT_LEN3) | (1<<SBIT_LEN4);

/* Enable the PWM output pins for PWM_1-PWM_4(P2_0 - P2_3) */

LPC_PWM1->PCR = (1<<SBIT_PWMENA1) | (1<<SBIT_PWMENA2) | (1<<SBIT_PWMENA3) | (1<<SBIT_PWMENA4);

while(1)

{

for(dutyCycle=0; dutyCycle<100; dutyCycle++)

{

LPC_PWM1->MR1 = dutyCycle;  /* Increase the dutyCycle from 0-100 */

LPC_PWM1->MR2 = dutyCycle;

LPC_PWM1->MR3 = dutyCycle;

LPC_PWM1->MR4 = dutyCycle;

/* Trigger the latch Enable Bits to load the new Match Values */

LPC_PWM1->LER = (1<<SBIT_LEN0) | (1<<SBIT_LEN1) | (1<<SBIT_LEN2) | (1<<SBIT_LEN3) | (1<<SBIT_LEN4);

delay_ms(5);

}

for(dutyCycle=100; dutyCycle>0; dutyCycle--)

{

LPC_PWM1->MR1 = dutyCycle;  /* Decrease the dutyCycle from 100-0 */

LPC_PWM1->MR2 = dutyCycle;

LPC_PWM1->MR3 = dutyCycle;

LPC_PWM1->MR4 = dutyCycle;

/* Trigger the latch Enable Bits to load the new Match Values */

LPC_PWM1->LER = (1<<SBIT_LEN0) | (1<<SBIT_LEN1) | (1<<SBIT_LEN2) | (1<<SBIT_LEN3) | (1<<SBIT_LEN4);

delay_ms(5);

}

}

}

Output:

Go to logic Analyzer (in View Panel)  setupadd pins by select add pin icon ‘port 2.1,2.2,2.3…’ and start analyser to see the below output. And disable and enable pin 11 to blink the LED. Change the display type to ‘bit’ in dropdown.

5. Control an LED using switch by polling method

#include <lpc17xx.h>

#define SwitchPinNumber 11

#define LedPinNumber    2

/* start the main program */

int main()

{

uint32_t switchStatus;

SystemInit();                             /* Clock and PLL configuration  */

LPC_PINCON->PINSEL4 = 0x000000;           /* Configure the Pins for GPIO  */

/* Configure the LED pin as output and SwitchPin as input */

LPC_GPIO2->FIODIR = ((1<<LedPinNumber) | (0<<SwitchPinNumber));

while(1)

{

/* Turn On all the leds and wait for one second */

switchStatus = (LPC_GPIO2->FIOPIN>>SwitchPinNumber) & 0x01 ;  /* Read the switch status */

if(switchStatus == 1)                        /* Turn ON/OFF LEDs depending on switch status */

{

LPC_GPIO2->FIOPIN = (1<<LedPinNumber);

}

else

{

LPC_GPIO2->FIOPIN = (0<<LedPinNumber);

}

}

}

Output:

Mask pin 11(tick in FIO2MASK Column) to switch on/off the LED . Go to peripheralsGPIO PinsPin2

6. Control an LED using switch by interrupt method

#include <lpc17xx.h>

#define PINSEL_EINT0    20

#define PINSEL_EINT1    22

#define LED1            0

#define LED2            1

#define SBIT_EINT0      0

#define SBIT_EINT1      1

#define SBIT_EXTMODE0   0

#define SBIT_EXTMODE1   1

#define SBIT_EXTPOLAR0  0

#define SBIT_EXTPOLAR1  1

void EINT0_IRQHandler(void)

{

LPC_SC->EXTINT = (1<<SBIT_EINT0);  /* Clear Interrupt Flag */

LPC_GPIO2->FIOPIN ^= (1<< LED1);   /* Toggle the LED1 everytime INTR0 is generated */

}

void EINT1_IRQHandler(void)

{

LPC_SC->EXTINT = (1<<SBIT_EINT1);  /* Clear Interrupt Flag */

LPC_GPIO2->FIOPIN ^= (1<< LED2);   /* Toggle the LED2 everytime INTR1 is generated */

}

int main()

{

SystemInit();

LPC_SC->EXTINT      = (1<<SBIT_EINT0)    | (1<<SBIT_EINT1);	    /* Clear Pending interrupts */

LPC_PINCON->PINSEL4 = (1<<PINSEL_EINT0)  | (1<<PINSEL_EINT1);   /* Configure P2_10,P2_11 as EINT0/1 */

LPC_SC->EXTMODE     = (1<<SBIT_EXTMODE0) | (1<<SBIT_EXTMODE1);  /* Configure EINTx as Edge Triggered*/

LPC_SC->EXTPOLAR    = (1<<SBIT_EXTPOLAR0)| (1<<SBIT_EXTPOLAR0); /* Configure EINTx as Falling Edge */

LPC_GPIO2->FIODIR   = (1<<LED1)  | (1<<LED2);                   /* Configure LED pins as OUTPUT */

LPC_GPIO2->FIOPIN   =  0x00;

NVIC_EnableIRQ(EINT0_IRQn);    /* Enable the EINT0,EINT1 interrupts */

NVIC_EnableIRQ(EINT1_IRQn);

while(1)

{

// Do nothing

}

}

Output:

Go to peripheralsGPIO PinsPin2

7. Display “Hello World” message using Internal UART

#include<LPC17xx.h>

void delay(unsigned int r1);

void UART0_Init(void);

void UART0_IRQHandler(void);

unsigned long int r=0, i = 0;

unsigned char tx0_flag=0;

unsigned char *ptr, arr[] = "Hello world";

int main(void)

{

SystemInit();

SystemCoreClockUpdate();

UART0_Init();

while(1)

{

ptr = arr;

while ( *ptr != '\0'){

LPC_UART0->THR = *ptr++;

while(tx0_flag == 0x00);

tx0_flag = 0x00;

for (i=0; i<200; i++);

}

for (i=0; i<500; i++)

delay(625);   //delay

}

}

void UART0_Init(void)

{

LPC_SC->PCONP |= 0x00000008;   //UART0 peripheral enable

LPC_PINCON->PINSEL0 |= 0x00000050;  //for selecting TX0[P0.2-->5:4] and RX0[P0.3-->7:6] of UART0

LPC_UART0->LCR = 0x00000083;  //enable divisor latch, parity disable, 1 stop bit, 8bit word length line control register

LPC_UART0->DLM = 0X00;

LPC_UART0->DLL = 0x13;         //select baud rate 9600 bps

LPC_UART0->LCR = 0X00000003;

LPC_UART0->FCR = 0x07;

LPC_UART0->IER = 0X03;       //select Transmit and receive interrupt

NVIC_EnableIRQ(UART0_IRQn); //Assigning channel

}

void UART0_IRQHandler(void)

{

unsigned long Int_Stat;

Int_Stat = LPC_UART0->IIR; //reading the data from interrupt identification register

Int_Stat = Int_Stat & 0x06; //masking other than txmit int & rcve data indicator

if((Int_Stat & 0x02)== 0x02) //transmit interrupt

tx0_flag = 0xff;

}

void delay(unsigned int r1)

{

for(r=0;r<r1;r++);

}

Output:

Go to View  Serial Window UART #1